Technical  Specifications  for Aluminium/Magnesium  Stir Casting furnace – 1 No

Aluminium/Magnesium Stir Casting furnace capacity minimum 1 Kg to maximum 5 Kg. Max. Temperature 1250 Deg.C.

Fast heating rate : furnace to attain 850 deg.C. in 20 minutes and 1200 deg. C in 50 minutes

Fast cooling rate : Provision to cool the melt at various cooling rate

Inner chamber size required  225 x 225 x 250 mm

Suitable stirrer mechanism to stir the melt in various predetermined speed up to 1500 rpm with DC motor and DC Drive. Impeller/Blade should be easily replaceable.

Suitable non-stick coating should be provided  for blades to avoid corrosion

Automatic up and down movement of stirrer assembly  and automatic movement at the top to make way for loading and unloading the charge into the furnace.

Split type door for opening, insertion of stirrer rod, Sensor and outlet of cooling media and closing.

Furnace outer shell should be made of Corrosive resistant stainless steel.

Control panel with PID Temperature controller for furnace temperature, another PID temperature controller for melt, Digital Ammeter, Digital Voltmeter, Digital RPM Indicator,

Stirrer assembly Up/Down Switch, DC Drive to control stirrer speed etc. to be supplied.

--------------------------------------------------------------------------------------

Any Technical Clarification in this regard please contact
Dr. Manas Mukherjee
Assistant Professor
Metallurgical and Materials Engineering
Indian Institute of Technology Madras
Chennai 600 036, India

Phone: +91-44-2257-4782/mail manas.mukherjee